Bosh BME280 TPH sensor

- I²C Wiring

- Connect Vin to the power supply, 3-5V is fine. Use the same voltage that the microcontroller logic is based off of. For most Arduinos, that is 5V

- Connect GND to common power/data ground

- Connect the SCK pin to the I2C clock SCL pin on your Arduino. On an UNO & '328 based Arduino, this is also known as A5, on a Mega it is also known as digital 21 and on a Leonardo/Micro, digital 3

- Connect the SDI pin to the I2C data SDA pin on your Arduino. On an UNO & '328 based Arduino, this is also known as A4, on a Mega it is also known as digital 20 and on a Leonardo/Micro, digital 2

- SPI Wiring

- Connect Vin to the power supply, 3V or 5V is fine. Use the same voltage that the microcontroller logic is based off of. For most Arduinos, that is 5V

- Connect GND to common power/data ground

- Connect the SCK pin to Digital #13 but any pin can be used later

- Connect the SDO pin to Digital #12 but any pin can be used later

- Connect the SDI pin to Digital #11 but any pin can be used later

- Connect the CS pin Digital #10 but any pin can be used later

Particle sensor: SDSO11

- Deze sensor gebruikt UART

- Power requirement:

  - Power Voltage：4.7~5.3V

  - Power supply：>1W

  - Supply voltage ripple：<20mV

- Interface specifications

- The UART communication protocol

  - Bit rate ：9600

  - Data bit ：8

  - Parity bit：NO

  - Stop bit ：1

  - Data Packet frequency: 1Hz